Options for the HIGH-SPEED ROLL-UP DOOR, type “EFA-SRT® CR Premium”

Curtain types:

2 mm PVC-coated polyester fibre according to the FDA Regulations 175.300, 178.2010 and 178.3740 on the transport of dry food, in colours

- blue, similar to RAL 5002, antistatic, silicone-free

- orange, similar to RAL 2008, non-antistatic, silicone-free

- red, similar to RAL 3002, non-antistatic, silicone-free

- yellow, similar to RAL 1021, non-antistatic, silicone-free

Window:

Window made of soft PVC, not approved in accordance with FDA Regulations. Due to the high tightness of the door, this window is subject to more wear and tear. The antistatic behaviour of the door leaf is reduced by a PVC window.

Complete coverings:

- 45° bevelled

- 90° vertical up to ceiling (only possible between H + 460mm and H + 1100mm)

Safety:

- Electrical emergency opening via button on frame for release of motor break via UPS

- Max. 2 photo cells in door frame, radars and active infrared sensor element for safety

- Door light-line grid (available for doors with clear height >= 2,000mm)

Self-monitoring, TÜV approved Door light-line grid (EFA-TLG®), integrated absolutely protected in the side frames of the door. The light grid operates in the door closing line and generates a nearly fully covering infrared light curtain up to a height of 2.5m. Objects are detected without any contact and the door closing movement stops immediately. By this, reversing the door blade can be triggered much quicker. Contact edge and/or photo cells do not apply

Activator and displays:

- clean room-compatible pressure switch (AP or UP), pull switch, radars

- contact-less proximity switch

- red/green LED traffic light, clean room version

GMP clean room air lock

Delivery and assembly of 2 high-speed roll-up doors to the prepared substructure in a clean room air lock with interlocking.