HIGH-SPEED FOLDING DOOR, type “EFA-SFT® THERM” (2-winged)

Manufacture, delivery and installation of

High-speed folding door of the type “EFA-SFT® THERM”, 2-wing, with electro-mechanic drive,

2-winged, i.e. 4 door segments in total, 2:2 coupled and centrally divided. Wings can be opened up to 90 degrees on each side

The door system mainly consists of:

Door leaf elements made of galvanized, thermally separated sheet steel profiles with integrated 
GRP profiles (glass-fibre reinforced plastic profile), Door leaves horizontally divided up to … fields,

Door panel made of thermally separated and foamed EFA-THERM® laths (40 mm thick). 
Door frames and opener box galvanized as standard, wing frame profiles painted acc. to RAL 7042 (fine structure), foamed fillings painted acc. to RAL 9006. Those fillings are held in the frame in a weatherproof manner by rubber seals and special molding. This allows a quick exchange of the complete fields.

Door leaves are sealed towards each another as well as towards the vertical steel frames and the opener box, floor seal with PES-fabric, rubber profile sealings between the wing segments, main closing edge sealed with hollow chamber rubber, at the same time developed as finger pinch guard, no guide rails on the floor. Depending to the size of the surface a central floor guidance will be provided.

Universal joint technology for the even weight distribution over the whole working area.

OPENING SPEED:	up to approx. 2.0 m/s

Max. DOOR LEAF SPEED:	up to approx. 2.5 m/s

(depending on the door width)

CLOSING SPEED:	up to approx. 1.0 m/s

The MICROPROCESSOR CONTROL is installed in a separate switch cabinet made of galvanised steel, protection type IP 65. Connection to 230 V / 50 Hz power supply on-site.

The scope of delivery includes an electric Safety contact edge, self-monitoring according to DIN EN12453:

Regulations pursuant to DIN EN 13241-1 are complied with

Resistance to wind load pursuant to DIN EN 12424 up to class 2
Airborne sound insulation pursuant to DIN EN 7171 up to 21 dB(A)

(values depend on the door size and the equipment)

for clear opening dimensions

Width = ............... mm  x  Height = ............... mm

Manufacturer:

EFAFLEX Tor- und Sicherheitssysteme GmbH & Co. KG

www.efaflex.com