„Machine protection door“

HIGH-SPEED ROLL-UP DOOR, type “EFA-SRT®-MS USD”

Manufacture, delivery and installation of

High-speed roll-up door type “EFA-SRT®-MS”:

The high-speed door EFA-SRT®-MS USD is a Machine protection door - safety component with functional safety performance level “d” for safety relevant function according to DIN EN ISO 13849. It is suitable for use as a movable separating safety guard, and as a personal-retaining-facility with locking according to DIN EN ISO 14120.

The standard door curtain is to be made of 2 mm transparent flexible PVC with warning stripes and reinforcing wind buttons and suitable for locations where paint or adhesives are applied.

The high-performance controller with frequency-converter allows together with the functionally adjusted drive a very dynamic door operation. The looking device for secure “door closed” message (limit switch: Pilz) enables a connection to machines in Cat. 4 / PL e acc. to

DIN EN ISO 13849-1).

OPENING SPEED:	up to approx. 1.3 m/s

Max. DOOR LEAF SPEED:	up to approx. 1.8 m/s

(depending on the lifting height)

CLOSING SPEED:	up to approx. 0.8 m/s

Max. cycles per minute:			7

The MICROPROCESSOR CONTROL is installed together with the integrated frequency converter in a separate plastic switch cabinet, protection type IP 54. Connection to 3~/N/PE 400V power supply on-site.

The scope of delivery includes an electric Safety contact edge, self-monitoring according to DIN EN 12453: Its connection cable must be guided in a protective energy chain within the door frame.

Regulations pursuant to DIN EN 13241-1 are complied with;

for clear opening dimensions

Width = ............... mm  x  Height = ............... mm

If a site-specific execution according to an existing product specification is required, please provide us those documents for technical examination!

Manufacturer:

EFAFLEX Tor- und Sicherheitssysteme GmbH & Co. KG

www.efaflex.com