ACTIVATORS for operating industrial door systems

Delivery, assembly and installation (incl. electrical installation with function check) of:

Pull switch with time control for automatic closing after the end of an adjustable time element

Bracket for previously described pull switch with console length adjustable from 1,000 to 3,000 mm

Push button in robust surface-type with time control for automatic closing after the end of an adjustable time element

Light barrier as activator for fixing at the on-site substructure near the door

Motion detector in a particularly robust, weather-resistant industrial version with adjustable radiation field from 0 – 6 m, especially for industrial doors

Bracket for the safe and torsion-free installation of the motion sensor

Option:

Motion detector in especially robust, weather-resistant industrial version with adjustable radiation field from 0 – 6 m, particularly for industrial doors with integrated infrared presence detector for detecting objects which move or stand immediately in front of the door system (max. installation height: 5m)

Bracket for the safe and torsion-free installation of the motion detector

Induction control with evaluation unit in the switch cabinet, preassembled and completely wired

2-channel version with mutual blocking of two induction loops, which can be evaluated separately from each other. If laid accordingly, the keep-open-times can be minimised since the door can close immediately after having been passed.

Laying the induction loops including wet-grinding, proper and workmanlike casting, connecting to the evaluation unit within the switch cabinet and function test.

up to maximally 20 m length

Radio receiver, 2-channel, complete incl. antenna, fitted into the switch cabinet and wired

Radio transmitter in heavy industrial version, 8 commands

Handset transmitter for radio control

Type “A”	1-channel

Type “B”	2-channel

Type “C”	6-channel

The standard radio control is to be designed so that one door can be opened each time the button is pushed. The door is closed automatically by means of an adjustable time segment.

Air-lock control

Interlocking of two high-speed doors which are installed in a suitable distance from each other. The control is to be designed so that only one door can be opened in each case while the other door is closed in order to achieve a real air-lock effect.

Option:

Control expansion in the form of an “automatic transfer impulse”

The electric wiring is realised by means of air-lock controls, principally according to time and material effort upon demonstration of evidence!