HIGH-SPEED ROLL-UP DOOR, type „EFA-SRT-L/PREMIUM®“

Manufacture, delivery and installation of

High-speed roll-up door, type „EFA-SRT-L/Premium®“, with electro-mechanical high-performance drive unit for permanent industrial application.

For the weight balancing of the door leaf and for manually opening the door in case of power failure (according to DIN EN 12604), a special tension spring mechanism is to be integrated into the lateral frame. This durable and extremely maintenance-friendly counter acting system is obligatory. Constructions using torsion springs are not admissible; standard door leaf made of 2 or 3 mm thick PVC, fully transparent, laterally led and wound onto horizontally supported shaft; PVC door curtain generally dispose of a vertical warning strip. A maintenance-free door leaf tension must be provided in order to permanently keep the door leaf tensioned with at least 500 N. In addition, the lateral CURTAIN GUIDES with special guiding devices are to be developed so that a flawless winding up and unwinding is also ensured during wind load. Steel frame construction, completely sealed, sendizimir-galvanised as standard.

The DOOR is driven by a geared motor which has to be developed as high-frequency motor. The positions of the door are permanently detected by means of non-wearing, inductive proximity switches, with the limits being determined electronically. Electro-mechanical limit switches are not permissible for this.

OPENING SPEED:	up to approx. 2.0 m/sec.

Max. DOOR LEAF SPEED:	up to approx. 2.6 m/sec.

(depending on door size)

CLOSING SPEED:	up to approx. 0.75m/sec.

The MICROPROCESSOR CONTROL is installed together with the integrated frequency converter in a separate plastic switch cabinet, protection type IP 65. Connection to 230V / 50 Hz power supply on-site.

The scope of delivery includes an electric Safety contact edge, self-monitoring according to DIN EN 12453: The signal of the contact edge is transmitted via radio.

Regulations pursuant to DIN EN 13241-1 are complied with;

for clear opening dimensions

Width = ............... mm  x  Height = ............... mm

Manufacturer:

EFAFLEX Tor- und Sicherheitssysteme GmbH & Co. KG

www.efaflex.com