HIGH SPEED SPIRAL DOOR, type „EFA-SST®- L Secure“

Manufacture, delivery and installation of

High-speed spiral door, type „EFA-SST®-Secure“, with electro-mechanical high-performance door drive for heaviest, permanent industrial application.

The certification according to DIN V ENV 1627-1630:1999-04 by an independent institute enables
the door system to be used in areas with requirements up to resistance class 4 (WK4) or RC4 acc. DIN/TS 18194:2020 / DIN EN 1627.

The door system mainly consists of:

Self-supporting lateral steel frames; steel parts generally galvanized, spiral-shaped door guidance

The load is transmitted on both sides: For achieving this, a synchronised shaft will be installed. For the exact, smooth and low-noise guidance of the hinge straps, ball-bearing precision rolling units have to be used. A sufficiently dimensioned tension spring mechanism, ensuring the weight balancing of the door leaf according to the standard DIN EN 12604 and a manual opening of the door (e.g. in the case of a power failure) is installed in the door frames.

On both sides arranged locking mechanisms automatically lock the door after each closing process.

Door leaf made of double-walled special-profiles, which are fixed onto hinge straps and moved vertically (i.e. up or down), surface anodized (E6/EV1).

This SPIRAL BODY is designed to guide the laths of the door leaf completely without contact and thus without wear and with best possible noise reduction.

The door system is equipped with a LOCKING MECHANISM. An internal hand lever is used

for operation.

The DOOR is driven by a geared motor which has to be developed as high-frequency motor. The positions of the door are permanently detected by means of non-wearing, inductive proximity switches, with the limits being determined electronically.

OPENING SPEED:	up to approx. 1.0 m/s

(depending on door size)

CLOSING SPEED:	up to approx. 0.6 m/s

The MICROPROCESSOR CONTROL is installed together with the integrated frequency converter in a separate plastic switch cabinet, protection type IP 65. Connection to 230V and/or 400 V / 50 Hz power supply on-site.

The scope of delivery includes a DOOR  LIGHT-LINE GRID (EFA-TLG®), TÜV certified and exactly effective in the door closing line: The safety system is integrated completely protected into the lateral frames and generates a very tight light grid of infrared beams up to a height of 2.5 metres. Obstructions are detected without contact; the automatic closing movement is stopped immediately.

The following guidelines and standards are taken into account in the

construction, production and assembly of the door, in particular

DIN EN 13241-1, DIN EN 60825-4, VDE 0837 part 4 and

DIN V ENV 1627-1630:1999-04 (WK 4)

for clear opening dimensions

Width = ............... mm  x  Height = ............... mm

Manufacturer:

EFAFLEX Tor- und Sicherheitssysteme GmbH & Co. KG

www.efaflex.com